San Jose State University

Department of Computer Engineering

CMPE 200 Report

Assignment 2 Report

Title MIPS Instruction Set Architecture & Programming (2)

Semester: Fall 2023	Date: 09/16/2023

by

Name: Sai Kashyap Kurella		SID: 016018925

- SOURCE CODE

# mipstest.smd

# Test the following MIPS instructions.

# add, sub, and, or, slt, addi, lw, sw, beq, j

#       Assembly                  Description           Address Machine

main:   addi $2, $0, 5          # initialize $2 = 5     0       20020005

addi $3, $0, 12         # initialize $3 = 12    4       2003000c

addi $7, $3, -9         # initialize $7 = 3     8       2067fff7

or   $4, $7, $2         # $4 <= 3 or 5 = 7      c       00e22025

and  $5, $3, $4         # $5 <= 12 and 7 = 4    10      00642824

add  $5, $5, $4         # $5 = 4 + 7 = 11       14      00a42820

beq  $5, $7, end        # shouldn't be taken    18      10a7000a

slt  $4, $3, $4         # $4 = 12 < 7 = 0       1c      0064202a

beq  $4, $0, around     # should be taken       20      10800001

addi $5, $0, 0          # shouldn't execute     24      20050000

around: slt  $4, $7, $2         # $4 = 3 < 5 = 1        28      00e2202a

add  $7, $4, $5         # $7 = 1 + 11 = 12      2c      00853820

sub  $7, $7, $2         # $7 = 12 - 5 = 7       30      00e23822

sw   $7, 68($3)         # [80] = 7              34      ac670044

lw   $2, 80($0)         # $2 = [80] = 7         38      8c020050

j    end                # should be taken       3c      08000011

addi $2, $0, 1          # shouldn't execute     40      20020001

end:    sw   $2, 84($0)         # write adr 84 = 7      44      ac020054

j    main               # go back to beginning  48      08000c00

- TEST LOG

- MARS & MIPSASM SNAPSHOTS

- DISCUSSIONS

- Initialization:

At the beginning of the main section, there is variable initialization using addi instructions. Registers $2, $3, and $7 are initialized with values 5, 12, and 3, respectively.

- Logical and Arithmetic Operations:

or: The or instruction performs a bitwise OR operation between registers $7 and $2, storing the result in $4. In this case, it calculates 3 OR 5, resulting in 7.

and: The and instruction performs a bitwise AND operation between registers $3 and $4, storing the result in $5. It calculates 12 AND 7, resulting in 4.

add: The add instruction adds registers $5 and $4, storing the result back in $5. It calculates 4 + 7, resulting in 11.

slt: The slt instruction sets $4 to 1 if $3 is less than $4 (12 < 7). In this case, it sets $4 to 0.

- Branching:

beq: There are two beq (branch on equal) instructions. The first one checks if $5 is equal to $7 and branches to the end label if true. The second one checks if $4 is equal to 0 and branches to the around label if true.

- Label Usage:

Labels like around, end, and main are used as targets for branching instructions, providing control flow within the program.

- Memory Operations:

sw: The sw (store word) instruction stores the value of register $7 into memory at address offset 68 from the address in register $3.

lw: The lw (load word) instruction loads a word from memory at address offset 80 from the address in register $0 (which is typically used as a null register) into register $2.

- Jump Instructions:

j: The j (jump) instruction is used to unconditionally jump to the specified label. It is used to control the program flow, both at the end and main labels.

- Commented Instruction Addresses:

The comments indicate the memory addresses for each instruction in hexadecimal format.

- Execution Flow:

The code has conditional branches (beq) and unconditional jumps (j), which determine the execution flow of the program. The program starts at the main label and loops between the around and end labels.

- CONCLUSION

To sum up, this MIPS assembly code performs basic arithmetic and logic operations, utilizes branching for conditional execution, and interacts with memory through load and store operations. The specific behaviour and results of the code execution would depend on the initial values of registers and memory locations, as well as the branching conditions. The machine codes of the corresponding assembly codes and register values were observed on the MARS & MIPSASM assemblers which helped me in realizing how an assembly code is interpreted by the underlying CPU.