San Jose State University

Department of Computer Engineering

CMPE 200 Report

Assignment-7 Report

Title Pipelined MIPS Processor, I/O and Interfacing

Semester: Fall 2023		Date: 12/2/2023

by

Name: Sharath Chandra Mummadi		SID 017400019

Name: Jincy Jose Thottathil			SID: 016903978

Name: Sai Kashyap Kurella			SID: 016018925

INTRODUCTION

The purpose of this lab is to convert the fifteen-instruction 32-bit single-cycle MIPS processor into a five-stage, pipelined design as well as Interface the processor with a factorial accelerator and a general-purpose I/O unit using memory-mapped interface registers.

DESIGN METHODOLOGY

Single Cycle System on Chip

This assignment is divided into two major parts, a single cycle system on a chip (SoC) design, and a pipelined MIPS processor working with the SoC. The single cycle MIPS processor completed in previous assignment was included and the factorial accelerator was modified such that it allowed each multiplication and countdown to take a single clock cycle. The GPIO was added in order to easily interface with factorial unit and MIPS processor. The address coming from single cycle MIPS is pulled directly off the output from instruction memory and is decoded in system level decoder and the proper addresses are sent to GPIO and Factorial units.

Figure 1: Single Cycle MIPS microarchitecture

# Figure 2: SoC Interfacing with Single cycle processor

| Mips Control |  |  |  |
| --- | --- | --- | --- |
| Base Address | Address Range | R/W | Description |
| 0x00000000 | 0x00 - 0xFC | R/W | Data Memory |
| 0x00000800 | 0x00 - 0x0C | R/W | Factorial Accelerator |
| 0x00000900 | 0x00 - 0x0C | R/W | General Purpose I/O |

Table 1. MIPS Control

| Factorial Accelerator Memory Map |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Decoder Output |  |  |
| Address | R/W | Register Name | Bits | Bit Defination | WE1 | WE2 | RdSel |
| 00 | R/W | Data Input (n) | 31:4 | Unused |  |  |  |
|  |  |  | 3:0 | n[3:0] | 1 | 0 | 00 |
| 01 | R/W | Control Input (n) | 31:1 | Unused |  |  |  |
|  |  |  | 0 | Go Bit | 0 | 1 | 01 |
| 10 | R | Control Input (Done, Err) | 31:2 | Unused |  |  |  |
|  |  |  | 1 | Err bit | 0 | 0 | 10 |
| 11 | R | Data Output (Result) | 0 | Done bit | 0 | 0 | 10 |
|  |  |  | 31:0 | ans[31:0] | 0 | 0 | 11 |

Table 2. Factorial Accelerator Memory Map

| GPIO Memory Map |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Decoder Output |  |  |
| Address | R/W | Register Name | Bits | Bit Defination | WE2 | RdSet |
| 00 | R | Input1 | 31:0 | General Input | 0 | 00 |
| 01 | R | Input2 | 31:0 | General Input | 0 | 01 |
| 10 | R/W | Output1 | 31:0 | General output | 0 | 10 |
| 11 | R/W | Output2 | 31:0 | General output | 1 | 11 |

Table 3. GPIO Memory Map

Pipelined MIPS Processor

MIPS processor was modified, and 4 registers were added to the processor that resulted in 5 stages: Fetch, Decode, Execute, Memory and Writeback. With this addition of registers came a set of data and signal hazards that needed to be addressed.

The following diagram shows where the data hazards occur. It is evident that at least one stall cycle is required depending on the instruction. Nops were used to resolve this problem.

Figure 3: Pipeline diagram of a standard pipelined MIPS processor

Figure 4: Pipelined MIPS microarchitecture

# Figure 5: SoC Interfacing with Pipelined MIPS processor

# Figure 6: Pipelined MIPS processor schematic with control unit, Hazard unit and data path

Performance Analysis of hardware accelerated n!

The factorial program was compared to analyze the performance gains for the SoC and pipelined MIPS SOC. The following equations show the results along with graphs shown in figure. It was found that the hardware accelerated version was close to 14 times more efficient that the software version of the program.

Single Cycle MIPS:

ETMIPS=14n-14+13

Hardware accelerated:

EThardware=n+2

Performance gain single cycle:

perfhardwareperfMIPS= 1EThardware1ETMIPS

EThardwareETMIPS= 14n-1+13n+2= 14n-1n+2 ≈14

Factorial Input

Figure 7: Single Cycle MIPS Processor Performance

Pipelined MIPS:

ETMIPS= 14n-1+135

Performance gained after pipelining:

perfhardwareperfMIPS= 1EThardware1ETMIPS

EThardwareETMIPS= 14n-1+135(n+2)= 14n-15n+10 ≈2.8

Factorial Input

Figure 7: Pipelined MIPS Processor Performance

TEST RESULTS

The program resides in memory memfile. When the clock is triggered, the instructions are executed accordingly. Below figures show the results of the testbench waveforms for Single Cycle processor SoC and Pipelined Processor SoC for 5!  Respectively. All the simulation results were as expected for both Single Cycle on Chip and the Pipelined Processor.

Figure 8: Simulations waveforms of Single Cycle MIPS processor running 5! program

Figure 8: Simulations waveforms of Pipelined MIPS processor running 5! program

CONCLUSION

The SoC interface design was implemented with full integration of the pipelined MIPS processor, factorial unit and the GPIO module. This assignment gave a deeper insight into the process of implementing SoC design and validating it with a unit level testbench.

Below task were successfully accomplished:

- Drafted of pipelined MIPS microarchitecture schematic.

- Drafted of SoC interface schematic.

- Tables for MIPS control unit and memory maps for SoC interface.

- Performance analysis of hardware accelerated n!

- Unit-level (with interface wrappers) simulation

- Completed interface design for the SoC with the pipelined MIPS processor, the factorial unit and the simple GPIO.

COLLABORATION:

Collaborated with my teammate to draft MIPS microarchitecture schematic, Draft of SoC interface schematic, analysis of hardware accelerated n! and interface design for the SoC with the pipelined MIPS processor.

APPENDIX

Adder:

module adder (

input wire [31:0] a,

input wire [31:0] b,

output wire [31:0] y

);

assign y = a + b;

endmodule

ALU:

module alu (

input wire [2:0] op,

input wire [4:0] shamt,

input wire [31:0] a,

input wire [31:0] b,

output wire zero,

output reg [31:0] y

);

assign zero = (y == 0);

always @ (op, a, b) begin

case (op)

3'b000: y = a & b;

3'b001: y = a | b;

3'b010: y = a + b;

3'b110: y = a - b;

3'b111: y = (a < b) ? 1 : 0;

3'b100: y = b << shamt;

3'b101: y = b >> shamt;

endcase

end

endmodule

auxdec:

module auxdec (

input wire [1:0] alu_op,

input wire [5:0] funct,

output wire we_reg_lh,

output wire sel_reg_lh,

output wire sel_lh,

output wire [2:0] alu_ctrl

);

reg [5:0] ctrl;

assign {alu_ctrl,we_reg_lh,sel_reg_lh,sel_lh} = ctrl;

always @ (alu_op, funct) begin

case (alu_op)

2'b00: ctrl = 6'b010000; // ADD

2'b01: ctrl = 6'b110000; // SUB

default: case (funct)

6'b10_0100: ctrl = 6'b000000; // AND 0

6'b10_0101: ctrl = 6'b001000; // OR 1

6'b10_0000: ctrl = 6'b010000; // ADD 2

6'b10_0010: ctrl = 6'b110000; // SUB 6

6'b10_1010: ctrl = 6'b111000; // SLT 7

6'b00_1000: ctrl = 6'b011000; // JR 3

6'b00_0000: ctrl = 6'b100000; // SLL 4

6'b00_0010: ctrl = 6'b101000; // SRL 5

6'b01_1001: ctrl = 6'b000100; // MULTU

6'b01_0000: ctrl = 6'b000011; // MFHI

6'b01_0010: ctrl = 6'b000010; // MFLO

default: ctrl = 6'bxxxxxx;

endcase

endcase

end

endmodule

dmem:

module dmem (

input wire clk,

input wire we,

input wire [5:0] a,

input wire [31:0] d,

output wire [31:0] q

);

reg [31:0] ram [0:63];

integer n;

initial begin

for (n = 0; n < 64; n = n + 1) ram[n] = 32'hFFFFFFFF;

end

always @ (posedge clk) begin

if (we) ram[a] <= d;

end

assign q = ram[a];

endmodule

signext:

module signext (

input wire [15:0] a,

output wire [31:0] y

);

assign y = {{16{a[15]}}, a};

endmodule

regfile:

module regfile (

input wire clk,

input wire we,

input wire we_pc,

input wire [4:0] ra1,

input wire [4:0] ra2,

input wire [4:0] ra3,

input wire [4:0] wa,

input wire [31:0] pc,

input wire [31:0] wd,

output wire [31:0] rd1,

output wire [31:0] rd2,

output wire [31:0] rd3

);

reg [31:0] rf [0:31];

integer n;

initial begin

for (n = 0; n < 32; n = n + 1) rf[n] = 32'h0;

rf[29] = 32'h100; // Initialze $sp

end

always @ (posedge clk) begin

if(we_pc) rf[31] <= pc;

else if (we) rf[wa] <= wd;

end

assign rd1 = (ra1 == 0) ? 0 : rf[ra1];

assign rd2 = (ra2 == 0) ? 0 : rf[ra2];

assign rd3 = (ra3 == 0) ? 0 : rf[ra3];

endmodule

mux4:

module mux4 #(parameter WIDTH = 8) (

input wire [1:0] sel,

input wire [WIDTH-1:0] a,

input wire [WIDTH-1:0] b,

input wire [WIDTH-1:0] c,

input wire [WIDTH-1:0] d,

output wire [WIDTH-1:0] y

);

wire [WIDTH-1:0] cd;

wire [WIDTH-1:0] ab;

assign y = sel[1] ? (sel[0] ? d:c):( sel[0] ? b:a);

endmodule

mux2:

module mux2 #(parameter WIDTH = 8) (

input wire sel,

input wire [WIDTH-1:0] a,

input wire [WIDTH-1:0] b,

output wire [WIDTH-1:0] y

);

assign y = (sel) ? b : a;

endmodule

mult_unit:

module mult_unit(input wire clk,

input wire we,

input wire sel,

input wire [31:0] a,

input wire [31:0] b,

output wire [31:0]rd1);

reg [31:0] reglh[0:1];

reg [63:0] ab;

integer n;

initial begin

for (n = 0; n < 2; n = n + 1) reglh[n] = 32'h0;

end

always @ (posedge clk) begin

if (we)

begin

ab = a*b;

reglh[0] <= ab[31:0];

reglh[1] <= ab[63:32];

end

end

assign rd1 = reglh[sel];

endmodule

MEM_WB:

module MEM_WB_Reg (

input wire clk,

input wire [4:0] ra12m,

input wire [5:0] ctrl_sigm,

input wire [31:0] alu_outm,

input wire [31:0] rd_dm,

input wire [31:0] mult_dm,

output wire [4:0] ra12b,

output wire [31:0] alu_outb,

output wire [31:0] rd_d_mb,

output wire [5:0] ctrl_sigb,

output wire [31:0] mult_dm_b

);

reg [31:0] rf [0:2];

reg [4:0] bm;

reg [5:0] rc;

integer n;

initial begin

for (n = 0; n < 3; n = n + 1) rf[n] = 32'h0;

bm = 5'h0;

rc = 6'h0;

end

always @ (posedge clk) begin

rf[0] = alu_outm;

rf[1] = rd_dm;

rf[2] = mult_dm;

bm = ra12m;

rc = ctrl_sigm;

end

assign alu_outb = rf[0];

assign rd_d_mb = rf[1];

assign mult_dm_b = rf[2];

assign ctrl_sigb = rc;

assign ra12b = bm;

endmodule

maindec:

module maindec (

input wire [5:0] opcode,

output wire branch,

output wire jump,

output wire reg_dst,

output wire we_reg,

output wire alu_src,

output wire we_dm,

output wire dm2reg,

output wire [1:0] alu_op

);

reg [8:0] ctrl;

assign {branch, jump, reg_dst, we_reg, alu_src, we_dm, dm2reg, alu_op} = ctrl;

always @ (opcode) begin

case (opcode)

6'b00_0000: ctrl = 9'b0_0_1_1_0_0_0_10; // R-type

6'b00_1000: ctrl = 9'b0_0_0_1_1_0_0_00; // ADDI

6'b00_0100: ctrl = 9'b1_0_0_0_0_0_0_01; // BEQ

6'b00_0010: ctrl = 9'b0_1_0_0_0_0_0_00; // J

6'b00_0011: ctrl = 9'b0_1_0_1_0_0_0_00; //JAL

6'b10_1011: ctrl = 9'b0_0_0_0_1_1_0_00; // SW

6'b10_0011: ctrl = 9'b0_0_0_1_1_0_1_00; // LW

default: ctrl = 9'bx_x_x_x_x_x_x_xx;

endcase

end

endmodule

imem:

module imem (

input wire [5:0] a,

output wire [31:0] y

);

reg [31:0] rom [0:63];

initial begin

$readmemh ("memfile.dat", rom);

end

assign y = rom[a];

endmodule

IF_ID_Reg:

module IF_ID_Reg (

input wire clk,

input wire [31:0] instrf,

input wire [31:0] pc_4f,

input wire stallD,

input wire clr_PCSRC,

output wire [31:0] instrD,

output wire [31:0] pc_4D

);

reg [31:0] rf [0:1];

integer n;

initial begin

for (n = 0; n < 2; n = n + 1) rf[n] = 32'h0;

end

always @ (posedge clk) begin

if(!stallD)

begin

rf[0] = instrf;

rf[1] = pc_4f;

end

if(clr_PCSRC == 1 && stallD == 0)

for (n = 0; n < 2; n = n + 1) rf[n] = 32'h0;

end

assign pc_4D = rf[1];

assign instrD = rf[0];

endmodule

ID_EXE_Reg:

module ID_EXE_Reg (

input wire clk,

input wire [12:0] ctrl_sigd,

input wire [31:0] pc_4d,

input wire [4:0] ras,

input wire [4:0] ra1d,

input wire [4:0] ra2d,

input wire [4:0] shamntd,

input wire [31:0] sext_immd,

input wire [25:0] jump_offset,

input wire [31:0] rd1d,

input wire [31:0] rd2d,

//

input wire flushE,

output wire [12:0] ctrl_sige,

output wire [31:0] pc_4e,

output wire [4:0] ra1e,

output wire [4:0] ra2e,

output wire [4:0] ras_e,

output wire [4:0] shamnte,

output wire [31:0] rd1e,

output wire [31:0] rd2e,

output wire [31:0] sext_imme,

output wire [25:0] jump_offset_e

);

reg [31:0] rf [0:3];

reg [4:0] rf4 [0:3];

reg [12:0] rfc;

reg [25:0] rj;

integer n;

task reset_exec;

begin

for (n = 0; n < 4; n = n + 1) rf[n] = 32'h0;

for (n = 0; n < 4; n = n + 1) rf4[n] = 5'h0;

rfc = 13'h0;

rj = 26'h0;

end

endtask

initial begin

reset_exec;

end

always @ (posedge clk) begin

rf[0] = pc_4d; rf[1] = rd1d; rf[2] = rd2d; rf[3] = sext_immd;

rf4[0] = ra1d; rf4[1] = ra2d; rf4[2] = shamntd; rf4[3] = ras;

rfc = ctrl_sigd;

rj = jump_offset;

if(flushE == 1)

begin

reset_exec;

end

end

assign pc_4e = rf[0];

assign rd1e = rf[1];

assign rd2e = rf[2];

assign sext_imme = rf[3];

assign jump_offset_e = rj;

assign ra1e = rf4[0];

assign ra2e = rf4[1];

assign shamnte = rf4[2];

assign ras_e = rf4[3];

assign ctrl_sige = rfc;

endmodule

EXE_MEM_Reg:

module EXE_MEM_Reg (

input wire clk,

input wire zero,

input wire [6:0] ctrl_sige,

//input wire [31:0] pc_4e,

input wire [31:0] alu_oute,

input wire [31:0] rd2e,

input wire [4:0] ra12e,

output wire zerom,

output wire [6:0] ctrl_sigm,

//output wire [31:0] pc_4m,

output wire [31:0] alu_outm,

output wire [31:0] rd2m,

output wire [4:0] ra12m

);

reg [31:0] rf [0:2];

reg [6:0] rc;

reg [4:0] rr;

reg rz;

integer n;

initial begin

for (n = 0; n < 3; n = n + 1) rf[n] = 32'h0;

rc = 7'h0;

rr = 5'h0;

rz = 0;

end

always @ (posedge clk) begin

//rf[0] = pc_4e;

rf[1] = alu_oute;

rf[2] = rd2e;

rz = zero;

rc = ctrl_sige;

rr = ra12e;

end

//assign pc_4e = rf[0];

assign alu_outm = rf[1];

assign rd2m = rf[2];

assign zerom = rz;

assign ctrl_sigm = rc;

assign ra12m = rr;

endmodule

GPIO:

module GPIO(

input wire clk,

input wire we,

input wire [31:0] wd,

output wire [31:0] rd

);

reg [31:0] out;

reg [31:0] in;

initial

out = 32'h5;

always @ (posedge(clk))

if (we)

in = wd;

assign rd = out;

endmodule

dreg:

module dreg # (parameter WIDTH = 32) (

input wire clk,

input wire rst,

input wire stallF,

input wire [WIDTH-1:0] d,

output wire [WIDTH-1:0] q

);

reg [WIDTH-1:0] ireg;

always @ (posedge clk, posedge rst) begin

if (rst) ireg = 0;

else if(stallF == 0) ireg = d;

end

assign q = ireg;

endmodule

hazardunit:

module hazardunit (

input wire we_reg_b,

input wire [4:0] ra12b,

input wire we_reg_m,

input wire [4:0] ra12m,

input wire dm2reg_e,

input wire [4:0] rs1e,

input wire [4:0] ra1e,

input wire [4:0] rs1d,

input wire [4:0] rt1d,

input wire jmp,

input wire we_reg_e,

input wire dm2reg_m,

input wire [4:0] ra12e,

input wire branchD,

output wire stallF,

output wire stallD,

output wire flushE,

output wire forwardAD,

output wire forwardBD,

output wire [1:0] forwardAE,

output wire [1:0] forwardBE

);

reg lwstall;

reg branchStall;

reg [1:0] fa;

reg [1:0] fb;

reg fBd;

reg fAd;

initial begin

lwstall = 0;

fa = 2'b0;

fb = 2'b0;

fAd = 0;

fBd = 0;

branchStall = 0;

end

always @(*) begin

lwstall = ((rs1d == ra1e || rt1d == ra1e) && dm2reg_e) ? 1:0;

fAd = ((rs1d != 0) && rs1d == ra12m && we_reg_m);

fBd = ((rt1d != 0) && rt1d == ra12m && we_reg_m);

// branchstall =

//BranchD AND RegWriteE AND (WriteRegE ==rsD OR WriteRegE ==rtD)

//OR

//BranchD AND MemtoRegM AND (WriteRegM ==rsD OR WriteRegM ==rtD)

branchStall = (branchD && we_reg_e && (ra12e == rs1d || ra12e == rt1d)) ||

(branchD && dm2reg_m && (ra12m == rs1d || ra12m == rt1d));

if ((rs1e != 0) && (rs1e == ra12m) && we_reg_m)

fa = 2'b10;

else if ((rs1e != 0) && (rs1e == ra12b) && we_reg_b)

fa = 2'b01;

else

fa = 2'b00;

if ((ra1e != 0) && (ra1e == ra12m) && we_reg_m)

fb = 2'b10;

else if ((ra1e != 0) && (ra1e == ra12b) && we_reg_b)

fb = 2'b01;

else

fb = 2'b00;

end

assign stallD = lwstall || branchStall;

assign stallF = lwstall || branchStall;

assign flushE = lwstall || branchStall;

assign forwardAE = fa;

assign forwardBE = fb;

assign forwardAD = fAd;

assign forwardBD = fBd;

endmodule

Datapath:

module datapath (

input wire clk,

input wire rst,

input wire branch,

input wire jump,

input wire reg_dst,

input wire we_reg,

input wire alu_src,

input wire dm2reg,

input wire we_reg_lh,

input wire sel_reg_lh,

input wire sel_lh,

input wire we_dm,

input wire [2:0] alu_ctrl,

input wire [4:0] ra3,

input wire [31:0] rd_dm,

//

input wire stallF,

input wire stallD,

input wire flushE,

input wire [1:0] forwardAE,

input wire [1:0] forwardBE,

input wire forwardAD,

input wire forwardBD,

//

output wire dm2reg_m,

output wire jmp,

output wire [31:0] instr_out,

output wire [31:0] pc_current,

output wire [31:0] alu_out,

output wire [31:0] wd_dm,

output wire [31:0] rd3,

//Hazard

output wire branchD,

output wire we_reg_e,

output wire [4:0] ra12e,

output wire we_reg_b,

output wire [4:0] ra12b,

output wire we_reg_m,

output wire [4:0] ra12m,

output wire dm2reg_e,

output wire [4:0] rs1e,

output wire [4:0] ra1e,

output wire [4:0] rs1d,

output wire [4:0] rt1d

);

wire [4:0] rf_wa;

wire [4:0] shamnt;

wire [31:0] pc_plus4;

wire [31:0] pc_pre;

wire [31:0] pc_next;

wire [31:0] sext_imm;

wire [31:0] ba;

wire [31:0] bta;

wire [31:0] jta;

wire [31:0] alu_pa;

wire [31:0] alu_pb;

wire [31:0] wd_rf;

wire [31:0] pre_pc_next;

wire [31:0] multu_lh;

wire [31:0] p_wd_rf;

wire [31:0] instr;

wire [31:0] pc_4D;

wire zero;

wire pc_src;

wire we_pc;

wire we_register;

wire jr_jmp;

wire [12:0] ctrl_sigd;

wire [12:0] ctrl_sige;

wire [31:0] pc_4e;

//wire [4:0] ra1e;

wire [4:0] ra2e;

wire [4:0] shamnte;

wire [31:0] rd1e;

wire [31:0] rd2e;

wire [31:0] sext_imme;

wire [25:0] jump_offset_e;

wire clr_PCSRC;

wire write_enable_data_memory;

wire write_enable_dev;

wire we_dev_fact;

wire we_dev_gpio;

wire [31:0] fact_out;

wire [31:0] gpio_out;

wire [31:0] mux_out_rd_dm;

wire zerom;

wire [6:0] ctrl_sigm;

//wire [31:0] pc_4m;

wire [31:0] alu_outm;

wire [31:0] rd2m;

//wire [4:0] ra12m;

wire [5:0] ctrl_sigb;

//wire [4:0] ra12b;

wire [31:0] alu_outb;

wire [31:0] rd_d_mb;

///////

wire [31:0] mux_out_fa;

wire [31:0] mux_out_fb;

wire [31:0] multu_lh_mb;

//

wire branchEqualD;

wire [31:0] rd1_alupmx;

wire [31:0] rd2_alupmx;

assign branchEqualD = (rd1_alupmx == rd2_alupmx) ? 1:0;

assign jmp = jump;

//

//

assign write_enable_data_memory = ctrl_sigm[0] && ~alu_outm[9];

assign write_enable_dev = ctrl_sigm[0] && alu_outm[9];

assign we_dev_fact = write_enable_dev && ~alu_outm[8];

assign we_dev_gpio = write_enable_dev && alu_outm[8];

//

assign clr_PCSRC = pc_src || jump || jr_jmp;

//

assign we_reg_b = ctrl_sigb[3];

assign we_reg_m = ctrl_sigm[4];

assign we_reg_e = ctrl_sige[9];

assign dm2reg_e = ctrl_sige[7];

assign dm2reg_m = ctrl_sigm[3]; //verify

assign ra12e = rf_wa;

assign rs1d = instr_out[25:21];

assign rt1d = instr_out[20:16];

//

//assign pc_src = ctrl_sigm[5] & zerom;

assign pc_src = branch & branchEqualD;

assign we_pc = ctrl_sige[11] & ctrl_sige[9];

assign we_register = ctrl_sigb[3] && ~ctrl_sigb[1];

assign ba = {sext_imm[29:0], 2'b00};

assign jta = {pc_4D[31:28], instr_out[25:0], 2'b00};

//assign jr_jmp = (ctrl_sige[2:0] == 3'b011) ? 1 : 0;

assign jr_jmp = (alu_ctrl == 3'b011) ? 1 : 0;

assign shamnt = instr_out[10:6];

assign branchD = branch;

assign ctrl_sigd = {branch, jump, reg_dst, we_reg, alu_src,

dm2reg, we_reg_lh,

sel_reg_lh, sel_lh, we_dm, alu_ctrl};

//Intermediate Register

IF_ID_Reg IF_ID_Reg(

.clk (clk),

.instrf (instr),

.pc_4f (pc_plus4),

.clr_PCSRC (clr_PCSRC),

.stallD (stallD),

.instrD (instr_out),

.pc_4D (pc_4D)

);

ID_EXE_Reg ID_EXE_Reg(

.clk (clk),

.ctrl_sigd (ctrl_sigd),

.pc_4d (pc_4D),

.ras (instr_out[25:21]),

.ra1d (instr_out[20:16]),

.ra2d (instr_out[15:11]),

.rd1d (alu_pa),

.rd2d (wd_dm),

.sext_immd (sext_imm),

.shamntd (shamnt),

.jump_offset (instr_out[25:0]),

.flushE (flushE),

.ctrl_sige (ctrl_sige),

.pc_4e (pc_4e),

.ra1e (ra1e),

.ra2e (ra2e),

.shamnte (shamnte),

.rd1e (rd1e),

.rd2e (rd2e),

.sext_imme (sext_imme),

.jump_offset_e (jump_offset_e),

.ras_e (rs1e)

);

EXE_MEM_Reg EXE_MEM_Reg(

.clk (clk),

.zero (zero),

.ctrl_sige ({ctrl_sige[4],ctrl_sige[12],ctrl_sige[9],ctrl_sige[7],

we_pc,ctrl_sige[5],ctrl_sige[3]}),

//.pc_4e (bta),

.alu_oute (alu_out),

.rd2e (mux_out_fb),

.ra12e (rf_wa),

.zerom (zerom),

.ctrl_sigm (ctrl_sigm),

//.pc_4m (pc_4m),

.alu_outm (alu_outm),

.rd2m (rd2m),

.ra12m (ra12m)

);

MEM_WB_Reg MEM_WB_Reg(

.clk (clk),

.ra12m (ra12m),

.ctrl_sigm

({ctrl_sigm[1],ctrl_sigm[6],ctrl_sigm[4],ctrl_sigm[3],ctrl_sigm[2],ctrl_sigm[5]}),

.alu_outm (alu_outm),

.rd_dm (mux_out_rd_dm),

.mult_dm (multu_lh),

.ra12b (ra12b),

.alu_outb (alu_outb),

.rd_d_mb (rd_d_mb),

.ctrl_sigb (ctrl_sigb),

.mult_dm_b (multu_lh_mb)

);

// --- PC Logic --- //

dreg pc_reg (

.clk (clk),

.rst (rst),

.stallF (stallF),

.d (pc_next),

.q (pc_current)

);

adder pc_plus_4 (

.a (pc_current),

.b (32'd4),

.y (pc_plus4)

);

adder pc_plus_br (

.a (pc_4D),

.b (ba),

.y (bta)

);

mux2 #(32) pc_src_mux (

.sel (pc_src),

.a (pc_plus4),

.b (bta),

.y (pc_pre)

);

mux2 #(32) pc_jmp_mux (

//.sel (ctrl_sige[11]),

.sel (jump),

.a (pc_pre),

.b (jta),

.y (pre_pc_next)

);

mux2 #(32) pc_jr_mux(

.sel (jr_jmp),

.a (pre_pc_next),

.b (alu_pa),

.y (pc_next)

);

//MEM

imem imem (

.a (pc_current[7:2]),

.y (instr)

);

dmem dmem (

.clk (clk),

.we (write_enable_data_memory),

.a (alu_outm[7:2]),

.d (rd2m),

.q (rd_dm)

);

//-------------------

//GPIO and FACTORIAL

FactTop ftp(

.clk (clk),

.reset (0),

.we (we_dev_fact),

.wa (alu_outm[1:0]),

.wd (rd2m[3:0]),

.rd (fact_out)

);

GPIO gpio(

.clk (clk),

.we (we_dev_gpio),

.wd (rd2m),

.rd (gpio_out)

);

mux4 #(32) mux_fact_gpio (

.sel (alu_outm[9:8]),

.a (rd_dm),

.b (rd_dm),

.c (fact_out),

.d (gpio_out),

.y (mux_out_rd_dm)

);

//------------------------

// --- RF Logic --- //

mux2 #(5) rf_wa_mux (

.sel (ctrl_sige[10]),

.a (ra1e),

.b (ra2e),

.y (rf_wa)

);

regfile rf (

.clk (clk),

.we (we_register),

.we_pc (we_pc),

.ra1 (instr_out[25:21]),

.ra2 (instr_out[20:16]),

.ra3 (ra3),

.wa (ra12b),

.pc (pc_4e),

.wd (wd_rf),

.rd1 (alu_pa),

.rd2 (wd_dm),

.rd3 (rd3)

);

//Branch Hazard

//After Decode Stage

mux2 #(32) rd1_palu(

.sel (forwardAD),

.a (alu_pa),

.b (alu_outm),

.y (rd1_alupmx)

);

mux2 #(32) rd2_palu(

.sel (forwardBD),

.a (wd_dm),

.b (alu_outm),

.y (rd2_alupmx)

);

signext se (

.a (instr_out[15:0]),

.y (sext_imm)

);

// --- ALU Logic --- //

mux2 #(32) alu_pb_mux (

.sel (ctrl_sige[8]),

.a (mux_out_fb),

.b (sext_imme),

.y (alu_pb)

);

//

mux4 #(32) mux_fa (

.sel (forwardAE),

.a (rd1e),

.b (wd_rf),

.c (alu_outm),

.d (32'b0),

.y (mux_out_fa)

);

mux4 #(32) mux_fb (

.sel (forwardBE),

.a (rd2e),

.b (wd_rf),

.c (alu_outm),

.d (32'b0),

.y (mux_out_fb)

);

//

alu alu (

.op (ctrl_sige[2:0]),

.shamt (shamnte),

.a (mux_out_fa),

.b (alu_pb),

.zero (zero),

.y (alu_out)

);

// --- MULTIPLICATION --- //

mult_unit mult_unit(

.clk (clk),

.we (ctrl_sige[6]),

.sel (ctrl_sigb[4]),

.a (mux_out_fa),

.b (alu_pb),

.rd1 (multu_lh)

);

// --- MEM Logic --- //

mux2 #(32) rf_wd_mux (

.sel (ctrl_sigb[2]),

.a (alu_outb),

.b (rd_d_mb),

.y (p_wd_rf)

);

mux2 #(32) rf_wd_reg_lh_mux(

.sel (ctrl_sigb[5]),

.a (p_wd_rf),

.b (multu_lh),

.y (wd_rf)

);

endmodule

Control unit:

module controlunit (

input wire [5:0] opcode,

input wire [5:0] funct,

output wire branch,

output wire jump,

output wire reg_dst,

output wire we_reg,

output wire alu_src,

output wire we_dm,

output wire dm2reg,

output wire we_reg_lh,

output wire sel_reg_lh,

output wire sel_lh,

output wire [2:0] alu_ctrl

);

wire [1:0] alu_op;

maindec md (

.opcode (opcode),

.branch (branch),

.jump (jump),

.reg_dst (reg_dst),

.we_reg (we_reg),

.alu_src (alu_src),

.we_dm (we_dm),

.dm2reg (dm2reg),

.alu_op (alu_op)

);

auxdec ad (

.alu_op (alu_op),

.funct (funct),

.we_reg_lh (we_reg_lh),

.sel_reg_lh (sel_reg_lh),

.sel_lh (sel_lh),

.alu_ctrl (alu_ctrl)

);

endmodule

module CNT(input wire clk,ld_cnt,en,

input wire [31:0] in,

output wire [31:0] _q);

reg [31:0]q;

initial

q = 0;

assign _q = q;

always @(posedge clk ) begin

if(ld_cnt)

q = in;

else if(en)

q = q - 1;

end

endmodule

module MUX(input wire sel,

input wire [31:0] a,b,

output wire [31:0] c);

assign c = sel == 0?a:b;

endmodule

module MUL(input wire [31:0] a,b,

output wire[31:0] ab);

assign ab = a * b;

endmodule

module CMP(input wire [31:0] a,b,

output wire gt);

assign gt = a>b?1:0;

endmodule

module REG(input wire clk,ld,

input wire [31:0] in,

output wire [31:0] _out);

reg [31:0] out;

initial

out = 0;

assign _out = out;

always @(posedge clk ) begin

if(ld)

out = in;

end

endmodule

module FDataPath(input wire clk,

input wire [4:0]control_signals,

input wire[31:0] in,

output wire [1:0] status_signals,

output wire [31:0] out);

wire [31:0] val_0,val_1,val_12;

assign val_0 = 0;

assign val_1 = 1;

assign val_12 = 12;

wire [31:0] out_cnt1,out_reg1,out_mult,out_mux1;

CNT cnt1(clk,control_signals[0],control_signals[1],in,out_cnt1);

CMP cmp1(out_cnt1,val_1,status_signals[1]);

CMP cmp2(out_cnt1,val_12,status_signals[0]);

REG reg1(clk,control_signals[3],out_mux1,out_reg1);

MUL mult(out_reg1,out_cnt1,out_mult);

MUX mux1(control_signals[4],val_1,out_mult,out_mux1);

MUX mux_out(control_signals[2],val_0,out_reg1,out);

endmodule

FControlUnit:

module FControlUnit(input wire clk,go,reset,

input wire [1:0] status_signals,

output reg err,done,

output reg [4:0] control_signals);

reg [2:0] cs,ns;

initial

begin

cs = 0;

err = 0;

done = 0;

end

always @(posedge clk ) begin

cs <= ns;

if(reset == 1)

begin

cs =0;

err =0;

done = 0;

end

end

always @(cs,go,status_signals) begin

case (cs)

3'b000:

begin

if(go)

ns = 1;

else

ns = 0;

control_signals = 0;

done = 0;

err = 0;

end

3'b001:

begin

control_signals = 5'b00001;

ns = 3'b010;

end

3'b010:

begin

if(status_signals[0])

begin

control_signals = 5'b00000;

ns = 3'b011;

end

else

begin

control_signals = 5'b01000;

ns = 3'b100;

end

end

3'b011:

begin

control_signals = 5'b00000;

ns = 3'b000;

done = 1;

err = 1;

end

3'b100:

begin

if(status_signals[1])

begin

control_signals = 5'b10000;

ns = 3'b101;

end

else

begin

control_signals = 5'b00000;

ns = 3'b111;

end

end

3'b101:

begin

control_signals = 5'b11000;

ns = 3'b110;

end

3'b110:

begin

control_signals = 5'b10010;

ns = 3'b100;

end

3'b111:

begin

control_signals = 5'b00100;

done = 1;

ns = 3'b000;

end

default:

begin

ns = 0;

cs = 0;

control_signals = 0;

end

endcase

end

endmodule

module FactorialUnit(input wire clk,go,reset,

input wire [3:0] n,

output wire err,done,

output wire [31:0] fans);

wire [4:0] control_signals;

wire [1:0] status_signals;

FControlUnit cu1(clk,go,reset,status_signals,err,done,control_signals);

FDataPath dp1(clk,control_signals,{28'b0,n},status_signals,fans);

endmodule

FactTop:

module FactTop(

input wire clk,

input wire reset,

input wire we,

input wire [1:0]wa,

input wire [3:0]wd,

output wire [31:0] rd

);

reg err_o;

reg done_o;

wire er,dn;

wire [31:0] fans;

reg [31:0] ans;

reg [3:0] n;

reg go_out,go;

reg [31:0] ird;

initial begin

err_o = 0;

done_o = 0;

go = 0;

go_out = 0;

n = 0;

end

always @(posedge clk) begin

if(er)

err_o = 1;

if(dn)

done_o = 1;

if(we)

begin

go = we && wd[0];

if(wa == 2'b00)

begin

n = wd;

end

else if(wa == 2'b01)

begin

go_out = wd[0];

err_o = 0;

done_o = 0;

end

end

if(dn)

ans = fans;

end

always @(*)

begin

case(wa)

2'b00: ird = n;

2'b01: ird = {30'b0,err_o,go};

2'b10: ird = {31'b0,done_o};

2'b11: ird = ans;

default: ird = 32'bXX;

endcase

end

assign rd = ird;

FactorialUnit fu(

.clk (clk),

.go (go),

.reset (reset),

.n (n),

.err (er),

.done (dn),

.fans (fans)

);

endmodule

mips_top:

module mips_top (

input wire clk,

input wire rst,

input wire [4:0] ra3,

output wire we_dm,

output wire [31:0] pc_current,

output wire [31:0] alu_out,

output wire [31:0] wd_dm,

output wire [31:0] rd_dm,

output wire [31:0] rd3

);

wire [31:0] DONT_USE;

mips mips (

.clk (clk),

.rst (rst),

.ra3 (ra3),

.rd_dm (rd_dm),

.we_dm (we_dm),

.pc_current (pc_current),

.alu_out (alu_out),

.wd_dm (wd_dm),

.rd3 (rd3)

);

endmodule

MIPS:

module mips (

input wire clk,

input wire rst,

input wire [4:0] ra3,

input wire [31:0] rd_dm,

output wire we_dm,

output wire [31:0] pc_current,

output wire [31:0] alu_out,

output wire [31:0] wd_dm,

output wire [31:0] rd3

);

wire branch;

wire jump;

wire reg_dst;

wire we_reg;

wire alu_src;

wire dm2reg;

wire we_reg_lh;

wire sel_reg_lh;

wire sel_lh;

wire jmp;

//

wire stallF;

wire stallD;

wire flushE;

wire [1:0] forwardAE;

wire [1:0] forwardBE;

// -----

wire we_reg_b;

wire [4:0] ra12b;

wire we_reg_m;

wire [4:0] ra12m;

wire dm2reg_e;

wire [4:0] rs1e;

wire [4:0] ra1e;

wire [4:0] rs1d;

wire [4:0] rt1d;

wire we_reg_e;

wire dm2reg_m;

wire [4:0] ra12e;

wire branchD;

wire forwardAD;

wire forwardBD;

//

wire [2:0] alu_ctrl;

wire [31:0] instr;

datapath dp (

.clk (clk),

.rst (rst),

.branch (branch),

.jump (jump),

.reg_dst (reg_dst),

.we_reg (we_reg),

.alu_src (alu_src),

.dm2reg (dm2reg),

.we_reg_lh (we_reg_lh),

.sel_reg_lh (sel_reg_lh),

.sel_lh (sel_lh),

.we_dm (we_dm),

.alu_ctrl (alu_ctrl),

.ra3 (ra3),

.rd_dm (rd_dm),

.dm2reg_m (dm2reg_m),

.forwardAD (forwardAD),

.forwardBD (forwardBD),

.stallF (stallF),

.stallD (stallD),

.flushE (flushE),

.forwardAE (forwardAE),

.forwardBE (forwardBE),

.instr_out (instr),

.pc_current (pc_current),

.alu_out (alu_out),

.wd_dm (wd_dm),

.rd3 (rd3),

.jmp (jmp),

.we_reg_e (we_reg_e),

.ra12e (ra12e),

.branchD (branchD),

.we_reg_b (we_reg_b),

.ra12b (ra12b),

.we_reg_m (we_reg_m),

.ra12m (ra12m),

.dm2reg_e (dm2reg_e),

.rs1e (rs1e),

.ra1e (ra1e),

.rs1d (rs1d),

.rt1d (rt1d)

);

hazardunit hu(

.we_reg_b (we_reg_b),

.ra12b (ra12b),

.we_reg_m (we_reg_m),

.ra12m (ra12m),

.dm2reg_e (dm2reg_e),

.rs1e (rs1e),

.ra1e (ra1e),

.rs1d (rs1d),

.rt1d (rt1d),

.we_reg_e (we_reg_e),

.ra12e (ra12e),

.branchD (branchD),

.jmp (jmp),

.dm2reg_m (dm2reg_m),

.stallF (stallF),

.stallD (stallD),

.flushE (flushE),

.forwardAE (forwardAE),

.forwardBE (forwardBE),

.forwardAD (forwardAD),

.forwardBD (forwardBD)

);

controlunit cu (

.opcode (instr[31:26]),

.funct (instr[5:0]),

.branch (branch),

.jump (jump),

.reg_dst (reg_dst),

.we_reg (we_reg),

.alu_src (alu_src),

.we_dm (we_dm),

.dm2reg (dm2reg),

.we_reg_lh (we_reg_lh),

.sel_reg_lh (sel_reg_lh),

.sel_lh (sel_lh),

.alu_ctrl (alu_ctrl)

);

endmodule

module tb_mips_top;

reg clk;

reg rst;

wire we_dm;

wire [31:0] pc_current;

wire [31:0] alu_out;

wire [31:0] wd_dm;

wire [31:0] rd_dm;

wire [31:0] DONT_USE;

mips_top DUT (

.clk (clk),

.rst (rst),

.we_dm (we_dm),

.ra3 (5'h0),

.pc_current (pc_current),

.alu_out (alu_out),

.wd_dm (wd_dm),

.rd_dm (rd_dm),

.rd3 (DONT_USE)

);

task tick;

begin

clk = 1'b0; #5;

clk = 1'b1; #5;

end

endtask

task reset;

begin

rst = 1'b0; #5;

rst = 1'b1; #5;

rst = 1'b0;

end

endtask

integer i;

initial begin

reset;

while(pc_current != 32'h1C) tick;

tick;tick;tick;tick;tick;tick;

$finish;

end

endmodule